ROUTER0 CONFIG

1. Deny IoT to Admin/Servers and Management

! Enter global configuration mode

configure terminal

! Create the extended ACL for the IoT network

ip access-list extended IOT_INBOUND

! Deny any traffic from IoT to the Admin/Servers network

deny   ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

! Deny IoT from accessing router via SSH (port 22) or Telnet (port 23)

deny   tcp 192.168.30.0 0.0.0.255 any eq 22

deny   tcp 192.168.30.0 0.0.0.255 any eq 23

! Permit all other traffic (to the internet)

! This is crucial to avoid blocking legitimate IoT traffic

permit ip any any

! Apply the ACL to the IoT gateway interface

Int fa1/0

ip access-group IOT_INBOUND in

exit

2. Restrict User to Admin via Jump-Host

! Enter global configuration mode

configure terminal

! Create the extended ACL for the User network

ip access-list extended USER_INBOUND

! Permit traffic from any User to the specific Jump-Host

permit ip 192.168.20.0 0.0.0.255 host 192.168.10.10

! Deny traffic from any User to the rest of the Admin network

deny   ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

! Permit all other traffic (e.g., to the internet)

permit ip any any

! Apply the ACL to the User gateway interface

Int fa1/0

ip access-group USER_INBOUND in

exit

3. Deny Outside Access

! Enter global configuration mode

configure terminal

! Create the extended ACL for the external interface

ip access-list extended OUTSIDE_INBOUND

! Permit return traffic for connections initiated from inside the network

permit tcp any any established

! Deny common attack vectors and unsolicited traffic to private ranges

deny   ip any 192.168.0.0 0.0.255.255

deny   ip any 10.0.0.0 0.255.255.255

deny   ip any 172.16.0.0 0.15.255.255

! Explicitly permit any other legitimate traffic you need (e.g., VPN)

ip access-list extended OUTSIDE_INBOUND

permit udp any any eq 500

permit esp any any

permit ah any any

permit tcp any any eq 443

permit ip any any

! Apply the ACL to the external-facing interface

int fa0/0

ip access-group OUTSIDE_INBOUND in

exit