DHCP (Server0)

P server (Packet Tracer Server GUI or Linux isc-dhcpd)

Packet Tracer Server GUI (fast)

IP: 192.168.1.100 / mask 255.255.255.0 / gateway 192.168.1.1

Services → DHCP:

Pool name: SMEPOOL

Default gateway: 192.168.1.1

DNS: 8.8.8.8

Start IP: 192.168.1.21

Subnet mask: 255.255.255.0

Max users: 150

*****

ASA Firewall

enable

configure terminal

hostname ASA5506

enable password itnpa4

! ---- Outside interface (to ISP)

interface GigabitEthernet1/2

nameif outside

security-level 0

ip address 203.0.113.10 255.255.255.0

no shutdown

! ---- Transit interface (toward Router0)

interface GigabitEthernet1/1

nameif transit

security-level 50

ip address 10.0.0.2 255.255.255.252

no shutdown

! ---- Routing

route outside 0.0.0.0 0.0.0.0 203.0.113.1

route transit 192.168.1.0 255.255.255.0 10.0.0.1

! ---- NAT

object network OBJ_LAN

subnet 192.168.1.0 255.255.255.0

nat (transit,outside) dynamic interface

! Optional: static port-forward for internal web server

object network OBJ_WEB

host 192.168.1.100

nat (transit,outside) static interface service tcp 80 80

! ---- Perimeter ACL (incoming to outside)

access-list OUTSIDE_IN extended permit tcp any host 203.0.113.10 eq 80

access-list OUTSIDE_IN extended permit tcp host 198.51.100.10 host 203.0.113.10 eq 22

access-list OUTSIDE_IN extended permit icmp any host 203.0.113.10 echo

access-list OUTSIDE_IN extended deny ip any any

access-group OUTSIDE_IN in interface outside

! ---- Management access

ssh 192.168.1.0 255.255.255.0 transit

ssh 198.51.100.10 255.255.255.255 outside

username admin password itnpa4

write memory

*****

Switch0

hostname Switch0

enable secret itnpa4

! keep default: all ports in VLAN1, no mgmt IP

! Connect:

! Fa0/1 -> PC-Admin

! Fa0/2 -> PC-Employee

! Fa0/3 -> IoT Camera

! Fa0/4 -> Server (DHCP) .100

! Fa0/24 -> Router0 Fa1/0

*****

Troubleshooting

Since gigEth1/1 was unavailable I used Management1/1 for transit

ASA5506#conf t

ASA5506(config)#interface GigabitEthernet1/2

ASA5506(config-if)#nameif transit

ERROR: Name "transit" has been assigned to interface GigabitEthernet1/1

ASA5506(config-if)#nameif transit1/2

ASA5506(config-if)#security-level 50

ASA5506(config-if)#ip address 10.0.0.2 255.255.255.252

ASA5506(config-if)#no shutdown

% 10.0.0.0 overlaps with GigabitEthernet1/1

GigabitEthernet1/2: incorrect IP address assignment

ASA5506(config-if)#exit

ASA5506(config)#exit

ASA5506#conf t

ASA5506(config)#interface Management1/1

ASA5506(config-if)#nameif transit

ERROR: Name "transit" has been assigned to interface GigabitEthernet1/1

ASA5506(config-if)#exit

ASA5506(config)#interface gigabitethernet1/1

ASA5506(config-if)#no nameif

ASA5506(config-if)#shutdown

%LINK-5-CHANGED: Interface GigabitEthernet1/1, changed state to administratively down

ASA5506(config-if)#exit

ASA5506(config)#interface Management1/1

ASA5506(config-if)#nameif transit

INFO: Security level for "transit" set to 0 by default.

ASA5506(config-if)#security-level 50

ASA5506(config-if)#ip address 10.0.0.2 255.255.255.252

ASA5506(config-if)#no shutdown

% 10.0.0.0 overlaps with GigabitEthernet1/2

Management1/1: incorrect IP address assignment

ASA5506(config-if)#exit

ASA5506(config)#interface gigabitethernet1/2

ASA5506(config-if)#no ip address

ASA5506(config-if)#shutdown

ASA5506(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/2, changed state to down

exit

ASA5506(config)#exit

ASA5506#conf t

ASA5506(config)#interface Management1/1

ASA5506(config-if)#nameif transit

ASA5506(config-if)#security-level 50

ASA5506(config-if)#ip address 10.0.0.2 255.255.255.252

ASA5506(config-if)#no shutdown

%LINK-5-CHANGED: Interface Management1/1, changed state to down

ASA5506(config-if)#no shutdown

ASA5506(config-if)#end

ASA5506#write memory

Building configuration...

Cryptochecksum: 47e36f17 3bed3e87 2a154148 124c2ed6

2177  bytes copied in 2.813 secs (773 bytes/sec)

[OK]

ASA5506#ping 10.0.0.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

ASA5506#

ASA5506#ping 10.0.0.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.1, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 0/0/0 ms

ASA5506#ping 10.0.0.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 0/0/0 ms

Outside Attacker connected to GigabitEthernet1/3 on ASA